# CPE403 – Advanced Embedded Systems

# Design Assignment 1

DO NOT REMOVE THIS PAGE DURING SUBMISSION:

Name: Xianjie Cao

Email: caox2@unlv.nevada.edu

Github Repository link (root): https://github.com/c1029324620/Flat-White

Youtube Playlist link (root):  https://www.youtube.com/playlist?list=PLY90fbcjLcrnosJGw9U__oC1jxRqwp8w8

Follow the submission guideline to be awarded points for this Assignment.

Submit the following for all Assignments:

- In the document, for each task submit the modified or included code (from the base code) with highlights and justifications of the modifications. Also include the comments. If no base code is provided, submit the base code for the first task only.

- Create a private Github repository with a random name (no CPE/403, Lastname, Firstname). Place all labs under the root folder TIVAC, sub-folder named Assignment1, with one document and one video link file for each lab, place modified c files named as asng_taskxx.c.

- If multiple c files or other libraries are used, create a folder asng1_t01 and place these files inside the folder.

- The folder should have a) Word document (see template), b) source code file(s) with startup_ccs.c and other include files, c) text file with youtube video links (see template).

- Submit the doc file in canvas before the due date. The root folder of the github assignment directory should have the documentation and the text file with youtube video links.

- Organize your youtube videos as playlist under the name “cpe403”. The playlist should have the video sequence arranged as submission or due dates.

- Only submit pdf documents. Do not forget to upload this document in the github repository and in the canvas submission portal.

- Code for Tasks. for each task submit the modified or included code (from the base code) with highlights and justifications of the modifications. Also include the comments. If no base code is provided, submit the base code for the first task only. Use separate page for each task.

Task1:

#include <stdint.h>

#include <stdbool.h>

#include "inc/hw_memmap.h"

#include "inc/hw_types.h"

#include "driverlib/gpio.h"

#include "driverlib/pin_map.h"

#include "driverlib/sysctl.h"

#include "driverlib/uart.h"

#include "inc/hw_ints.h"

#include "driverlib/interrupt.h"

#include "driverlib/timer.h"

#include "driverlib/debug.h"

#include "driverlib/adc.h"

#include "utils/uartstdio.h"

#include <string.h>

#ifdef DEBUG

void__error__(char *pcFilename, uint32_t ui32Line)

{

}

#endif

// Globals

uint32_t ui32Period;

char     buffer[4];

uint8_t status = 0x0E;

uint32_t ui32ADC0Value[4];

volatile uint32_t ui32TempAvg;

volatile uint32_t ui32TempValueC;

volatile uint32_t ui32TempValueF;

// Timer 1 ISR

void Timer1IntHandler(void)

{

// Clear the timer interrupt

TimerIntClear(TIMER1_BASE, TIMER_TIMA_TIMEOUT);

ADCIntClear(ADC0_BASE, 2);

ADCProcessorTrigger(ADC0_BASE, 2);

ADCSequenceDataGet(ADC0_BASE, 2, ui32ADC0Value);

ui32TempAvg = (ui32ADC0Value[0] + ui32ADC0Value[1] + ui32ADC0Value[2] + ui32ADC0Value[3] + 2)/4;

ui32TempValueC = (1475 - ((2475 * ui32TempAvg)) / 4096)/10;

ui32TempValueF = ((ui32TempValueC * 9) + 160) / 5;

UARTprintf("C %3d\t",ui32TempValueC );

UARTprintf("F %3d\t",ui32TempValueF );

UARTprintf("\n");

}

void GPIOF4IntHandler(void)

{

//Toggle the RGB LED.

status = status ^ 14;

//clear GPIO interrupt

GPIOIntClear(GPIO_PORTF_BASE, GPIO_INT_PIN_4);

GPIOPinWrite(GPIO_PORTF_BASE,GPIO_PIN_1|GPIO_PIN_2|GPIO_PIN_3, status);

}

int main(void) {

// Configure Clock

SysCtlClockSet(SYSCTL_SYSDIV_5 | SYSCTL_USE_PLL | SYSCTL_OSC_MAIN | SYSCTL_XTAL_16MHZ);

// Configure peripherals

SysCtlPeripheralEnable(SYSCTL_PERIPH_UART0);

SysCtlPeripheralEnable(SYSCTL_PERIPH_GPIOA);

SysCtlPeripheralEnable(SYSCTL_PERIPH_TIMER1);  // Enabling Timer 1

SysCtlPeripheralEnable(SYSCTL_PERIPH_GPIOF);   //enable GPIOF port.

//Configure GPIO port F

GPIOPinTypeGPIOOutput(GPIO_PORTF_BASE, GPIO_PIN_1|GPIO_PIN_2|GPIO_PIN_3);

GPIOPinTypeGPIOInput(GPIO_PORTF_BASE, GPIO_PIN_4);

GPIOPadConfigSet(GPIO_PORTF_BASE, GPIO_PIN_4, GPIO_STRENGTH_2MA, GPIO_PIN_TYPE_STD_WPU);

GPIOIntRegister(GPIO_PORTF_BASE, GPIOF4IntHandler); //register hanndler function for port F

GPIOIntTypeSet(GPIO_PORTF_BASE, GPIO_PIN_4, GPIO_FALLING_EDGE); //PF4 for falling edge trigger.

GPIOIntEnable(GPIO_PORTF_BASE, GPIO_INT_PIN_4);     //enable interrupt for PF4.

// Configure ADC

SysCtlPeripheralEnable(SYSCTL_PERIPH_ADC0);

ADCHardwareOversampleConfigure(ADC0_BASE, 32);

ADCSequenceConfigure(ADC0_BASE, 2, ADC_TRIGGER_PROCESSOR, 0); // Changed to sequencer #2

ADCSequenceStepConfigure(ADC0_BASE, 2, 0, ADC_CTL_TS);

ADCSequenceStepConfigure(ADC0_BASE, 2, 1, ADC_CTL_TS);

ADCSequenceStepConfigure(ADC0_BASE, 2, 2, ADC_CTL_TS);

ADCSequenceStepConfigure(ADC0_BASE, 2, 3, ADC_CTL_TS|ADC_CTL_IE|ADC_CTL_END);

ADCSequenceEnable(ADC0_BASE, 2);

////////////////////

// Configure Timer 1 module

TimerConfigure(TIMER1_BASE, TIMER_CFG_PERIODIC);

ui32Period = SysCtlClockGet()/2;   // Period of 0.5s 2Hz

TimerLoadSet(TIMER1_BASE, TIMER_A, ui32Period -1);

IntEnable(INT_TIMER1A);

TimerIntEnable(TIMER1_BASE, TIMER_TIMA_TIMEOUT);

// Configure pins for UART

GPIOPinConfigure(GPIO_PA0_U0RX);

GPIOPinConfigure(GPIO_PA1_U0TX);

GPIOPinTypeUART(GPIO_PORTA_BASE, GPIO_PIN_0 | GPIO_PIN_1);

UARTClockSourceSet(UART0_BASE, UART_CLOCK_PIOSC);

UARTStdioConfig(0, 115200, 16000000);

// Enable interrupts

IntMasterEnable();

TimerEnable(TIMER1_BASE, TIMER_A);

ADCSequenceEnable(ADC0_BASE, 2);

// Initial message to terminal display

UARTprintf("Temperature:\n");

while (1)  // Wait forever

{

}

}

Task 2:

#include <stdint.h>

#include <stdbool.h>

#include "inc/hw_memmap.h"

#include "inc/hw_types.h"

#include "driverlib/gpio.h"

#include "driverlib/pin_map.h"

#include "driverlib/sysctl.h"

#include "driverlib/uart.h"

#include "inc/hw_ints.h"

#include "driverlib/interrupt.h"

#include "driverlib/timer.h"

#include "driverlib/debug.h"

#include "driverlib/adc.h"

#include "utils/uartstdio.h"

#include <string.h>

#ifdef DEBUG

void__error__(char *pcFilename, uint32_t ui32Line)

{

}

#endif

// Globals

uint32_t ui32Period;

char     buffer[4];

uint8_t status = 0x0E;

uint32_t ui32ADC0Value[4];

volatile uint32_t ui32TempAvg;

volatile uint32_t ui32TempValueC;

volatile uint32_t ui32TempValueF;

// Timer 1 ISR

void Timer1IntHandler(void)

{

// Clear the timer interrupt

TimerIntClear(TIMER1_BASE, TIMER_TIMA_TIMEOUT);

//ADC keep running.

ADCIntClear(ADC0_BASE, 2);

ADCProcessorTrigger(ADC0_BASE, 2);

ADCSequenceDataGet(ADC0_BASE, 2, ui32ADC0Value);

ui32TempAvg = (ui32ADC0Value[0] + ui32ADC0Value[1] + ui32ADC0Value[2] + ui32ADC0Value[3] + 2)/4;

ui32TempValueC = (1475 - ((2475 * ui32TempAvg)) / 4096)/10;

ui32TempValueF = ((ui32TempValueC * 9) + 160) / 5;

}

void display_tem_c()

{

UARTprintf("Temperature in C: %3d\n", ui32TempValueC);

}

void display_tem_f()

{

UARTprintf("Temperature in F: %3d\n", ui32TempValueF);

}

void red_on()

{

//turn RED LED on

GPIOPinWrite(GPIO_PORTF_BASE, GPIO_PIN_1, GPIO_PIN_1);

}

void blue_on()

{

//turn BLUE LED on

GPIOPinWrite(GPIO_PORTF_BASE, GPIO_PIN_2, GPIO_PIN_2);

}

void green_on()

{

//Turn GREEN LED on

GPIOPinWrite(GPIO_PORTF_BASE, GPIO_PIN_3, GPIO_PIN_3);

}

void red_off()

{

GPIOPinWrite(GPIO_PORTF_BASE, GPIO_PIN_1, 0x0);

}

void blue_off()

{

GPIOPinWrite(GPIO_PORTF_BASE, GPIO_PIN_2, 0x0);

}

void green_off()

{

GPIOPinWrite(GPIO_PORTF_BASE, GPIO_PIN_3, 0x0);

}

void get_status()

{

uint8_t red, green, blue;

red = GPIOPinRead(GPIO_PORTF_BASE, GPIO_PIN_1);

blue = GPIOPinRead(GPIO_PORTF_BASE, GPIO_PIN_2);

green = GPIOPinRead(GPIO_PORTF_BASE, GPIO_PIN_3);

if(red == 0x0)

UARTprintf("Red LED is OFF\n");

else

UARTprintf("Red LED is ON\n");

if(green == 0x0)

UARTprintf("Green LED is OFF\n");

else

UARTprintf("Green LED is ON\n");

if(blue == 0x0)

UARTprintf("Blue LED is OFF\n");

else

UARTprintf("Blue LED is ON\n");

}

int main(void)

{

//40MHz

SysCtlClockSet(SYSCTL_SYSDIV_5 | SYSCTL_USE_PLL | SYSCTL_OSC_MAIN | SYSCTL_XTAL_16MHZ);

SysCtlPeripheralEnable(SYSCTL_PERIPH_UART0);    //Enabling UART

SysCtlPeripheralEnable(SYSCTL_PERIPH_GPIOA);    //Enabling GPIO port A

SysCtlPeripheralEnable(SYSCTL_PERIPH_TIMER1);  // Enabling Timer 1

SysCtlPeripheralEnable(SYSCTL_PERIPH_GPIOF);   //enable GPIOF port.

// Configure ADC

SysCtlPeripheralEnable(SYSCTL_PERIPH_ADC0);

ADCHardwareOversampleConfigure(ADC0_BASE, 32);

GPIOPinTypeGPIOOutput(GPIO_PORTF_BASE, GPIO_PIN_1|GPIO_PIN_2|GPIO_PIN_3);

ADCSequenceConfigure(ADC0_BASE, 2, ADC_TRIGGER_PROCESSOR, 0); // Changed to sequencer #2

ADCSequenceStepConfigure(ADC0_BASE, 2, 0, ADC_CTL_TS);

ADCSequenceStepConfigure(ADC0_BASE, 2, 1, ADC_CTL_TS);

ADCSequenceStepConfigure(ADC0_BASE, 2, 2, ADC_CTL_TS);

ADCSequenceStepConfigure(ADC0_BASE, 2, 3, ADC_CTL_TS|ADC_CTL_IE|ADC_CTL_END);

ADCSequenceEnable(ADC0_BASE, 2);

// Configure Timer 1 module

TimerConfigure(TIMER1_BASE, TIMER_CFG_PERIODIC);

ui32Period = SysCtlClockGet()/2;   // Period of 0.5s 2Hz

TimerLoadSet(TIMER1_BASE, TIMER_A, ui32Period -1);

IntEnable(INT_TIMER1A);

TimerIntEnable(TIMER1_BASE, TIMER_TIMA_TIMEOUT);

// Configure pins for UART

GPIOPinConfigure(GPIO_PA0_U0RX);

GPIOPinConfigure(GPIO_PA1_U0TX);

GPIOPinTypeUART(GPIO_PORTA_BASE, GPIO_PIN_0 | GPIO_PIN_1);

UARTClockSourceSet(UART0_BASE, UART_CLOCK_PIOSC);

UARTStdioConfig(0, 115200, 16000000);

// Enable interrupts

IntMasterEnable();

TimerEnable(TIMER1_BASE, TIMER_A);

ADCSequenceEnable(ADC0_BASE, 2);

UARTprintf("***************Enter the command*****************\n");

UARTprintf("**R|r => Red LED  B|b => Blue LED G|g => Green LED**\n");

UARTprintf("***T => Temp in C | t => Temp in F | S => RGB Status***\n");

while(1)

{

switch(UARTCharGet(UART0_BASE))

{

case 'T':

display_tem_c();

break;

case 't':

display_tem_f();

break;

case 'R':

red_on();

break;

case 'r':

red_off();

break;

case 'B':

blue_on();

break;

case 'b':

blue_off();

break;

case 'G':

green_on();

break;

case 'g':

green_off();

break;

case 'S':

get_status();

break;

default:

break;

}

}

}

Task 3:

#include <stdbool.h>

#include <stdint.h>

#include "inc/hw_uart.h"

#include "inc/hw_ints.h"

#include "inc/hw_memmap.h"

#include "inc/hw_adc.h"

#include "inc/hw_types.h"

#include "inc/hw_udma.h"

#include "driverlib/debug.h"

#include "driverlib/gpio.h"

#include "driverlib/interrupt.h"

#include "driverlib/pin_map.h"

#include "driverlib/sysctl.h"

#include "driverlib/uart.h"

#include "driverlib/adc.h"

#include "driverlib/udma.h"

#include "driverlib/timer.h"

#include "driverlib/rom.h"

#include "driverlib/rom_map.h"

#include "driverlib/systick.h"

#include "utils/uartstdio.h"

#define ADC_SAMPLE_BUF_SIZE     64

enum BUFFERSTATUS

{ EMPTY,

FILLING,

FULL

};

#pragma DATA_ALIGN(ucControlTable, 1024)

uint8_t ucControlTable[1024];

static uint16_t ADC_Out1[ADC_SAMPLE_BUF_SIZE];

static uint16_t ADC_Out2[ADC_SAMPLE_BUF_SIZE];

static enum BUFFERSTATUS BufferStatus[2];

volatile uint32_t ui32Temp;

void ConfigureUART(void);

void init_ADC(void);

void init_TIMER(void);

void init_DMA(void);

static uint32_t g_ui32DMAErrCount = 0u;

static uint32_t g_ui32SysTickCount;

void uDMAErrorHandler(void)

{

uint32_t ui32Status;

ui32Status = MAP_uDMAErrorStatusGet();

if(ui32Status)

{

MAP_uDMAErrorStatusClear();

g_ui32DMAErrCount++;

}

}

// Not used in this example, but used to debug to make sure timer interrupts happen

void Timer0AIntHandler(void)

{

//

// Clear the timer interrupt flag.

//

TimerIntClear(TIMER0_BASE, TIMER_TIMA_TIMEOUT);

}

void SysTickIntHandler(void)

{

// Update our system tick counter.

g_ui32SysTickCount++;

}

void ADCseq0Handler()

{

ADCIntClear(ADC0_BASE, 0);

if ((uDMAChannelModeGet(UDMA_CHANNEL_ADC0 | UDMA_PRI_SELECT) == UDMA_MODE_STOP)

&& (BufferStatus[0] == FILLING))

{

BufferStatus[0] = FULL;

BufferStatus[1] = FILLING;

}

else if ((uDMAChannelModeGet(UDMA_CHANNEL_ADC0 | UDMA_ALT_SELECT) == UDMA_MODE_STOP)

&& (BufferStatus[1] == FILLING))

{

BufferStatus[0] = FILLING;

BufferStatus[1] = FULL;

}

}

int main(void)

{

uint32_t i, average1, average2, samples_taken;

// Set the system clock to run at 80MHz from the PLL.

SysCtlClockSet(SYSCTL_SYSDIV_2_5|SYSCTL_USE_PLL|SYSCTL_OSC_MAIN|SYSCTL_XTAL_16MHZ);

SysCtlDelay(20u);

MAP_SysCtlPeripheralEnable(SYSCTL_PERIPH_TIMER0);  //Enable the clock to TIMER0

MAP_SysCtlPeripheralEnable(SYSCTL_PERIPH_ADC0);    //Enable the clock to ADC module

MAP_SysCtlPeripheralEnable(SYSCTL_PERIPH_UDMA);    //Enable the clock to uDMA

MAP_SysCtlPeripheralEnable(SYSCTL_PERIPH_GPIOE);   //Enables the clock to PORT E

MAP_SysCtlDelay(30u);

MAP_SysTickPeriodSet(SysCtlClockGet() / 100000u); //Sets the period of the SysTic counter to 10us

MAP_SysTickIntEnable();

MAP_SysTickEnable();

BufferStatus[0] = FILLING;

BufferStatus[1] = EMPTY;

samples_taken = 0u;

ConfigureUART();

UARTprintf("ADC->uDMA->UART\n");

UARTprintf("\tTempC\tTempF\tTotal Samples\n");

init_DMA();

init_ADC();

init_TIMER();

MAP_IntMasterEnable();

MAP_TimerEnable(TIMER0_BASE, TIMER_A); // Start everything

while(1)

{

if(BufferStatus[0u] == FULL)

{

// Do something with data in ADC_OUT1

average1 = 0u;

for(i =0u; i < ADC_SAMPLE_BUF_SIZE; i++)

{

average1 += ADC_Out1[i];

ADC_Out1[i] = 0u;

}

BufferStatus[0u] = EMPTY;

// Enable for another uDMA block transfer

uDMAChannelTransferSet(UDMA_CHANNEL_ADC0 | UDMA_PRI_SELECT, UDMA_MODE_PINGPONG, (void *)(ADC0_BASE + ADC_O_SSFIFO0), &ADC_Out1, ADC_SAMPLE_BUF_SIZE);

uDMAChannelEnable(UDMA_CHANNEL_ADC0 | UDMA_PRI_SELECT); // Enables DMA channel so it can perform transfers

samples_taken += ADC_SAMPLE_BUF_SIZE;

average1 = (average1 + (ADC_SAMPLE_BUF_SIZE / 2u)) / ADC_SAMPLE_BUF_SIZE;

average1 = (1475 - ((2475 * average1)) / 4096)/10;      //temp in C

}

if(BufferStatus[1u] == FULL)

{

// Do something with data in ADC_OUT2

average2 = 0u;

for(i =0u; i < ADC_SAMPLE_BUF_SIZE; i++)

{

average2 += ADC_Out2[i];

ADC_Out2[i] = 0u;

}

BufferStatus[1u] = EMPTY;

// Enable for another uDMA block transfer

uDMAChannelTransferSet(UDMA_CHANNEL_ADC0 | UDMA_ALT_SELECT, UDMA_MODE_PINGPONG, (void *)(ADC0_BASE + ADC_O_SSFIFO0), &ADC_Out2, ADC_SAMPLE_BUF_SIZE);

uDMAChannelEnable(UDMA_CHANNEL_ADC0 | UDMA_ALT_SELECT);

samples_taken += ADC_SAMPLE_BUF_SIZE;

average2 = (average2 + (ADC_SAMPLE_BUF_SIZE / 2u)) / ADC_SAMPLE_BUF_SIZE;

average2 = ((average1 * 9) + 160) / 5;          //temp in F

UARTprintf("\t%d\t%d\t%d\r", average1,average2,samples_taken);

}

}

}

void init_TIMER()

{

MAP_TimerConfigure(TIMER0_BASE, TIMER_CFG_SPLIT_PAIR | TIMER_CFG_A_PERIODIC);

// Set sample frequency to 16KHz (every 62.5uS)

MAP_TimerLoadSet(TIMER0_BASE, TIMER_A, MAP_SysCtlClockGet()/16000 -1);   //TODO: Timer Load Value is set here

MAP_TimerControlTrigger(TIMER0_BASE, TIMER_A, true);

MAP_TimerControlStall(TIMER0_BASE, TIMER_A, true); //Assist in debug by stalling timer at breakpoints

}

void init_ADC()

{

SysCtlPeripheralEnable(SYSCTL_PERIPH_ADC0);

ADCHardwareOversampleConfigure(ADC0_BASE, 32);

SysCtlDelay(80u);

// Use ADC0 sequence 0 to sample channel 0 once for each timer period

ADCClockConfigSet(ADC0_BASE, ADC_CLOCK_SRC_PIOSC | ADC_CLOCK_RATE_HALF, 1);

SysCtlDelay(10); // Time for the clock configuration to set

IntDisable(INT_ADC0SS0);

ADCIntDisable(ADC0_BASE, 0u);

ADCSequenceDisable(ADC0_BASE,0u);

// With sequence disabled, it is now safe to load the new configuration parameters

ADCSequenceConfigure(ADC0_BASE, 0u, ADC_TRIGGER_TIMER, 0u);

ADCSequenceStepConfigure(ADC0_BASE,0,0, ADC_CTL_TS | ADC_CTL_END | ADC_CTL_IE);

ADCSequenceEnable(ADC0_BASE,0u); //Once configuration is set, re-enable the sequencer

ADCIntClear(ADC0_BASE,0u);

ADCSequenceDMAEnable(ADC0_BASE,0);

IntEnable(INT_ADC0SS0);

}

void init_DMA()

{

uDMAEnable(); // Enables uDMA

uDMAControlBaseSet(ucControlTable);

uDMAChannelAttributeDisable(UDMA_CHANNEL_ADC0, UDMA_ATTR_ALTSELECT | UDMA_ATTR_HIGH_PRIORITY | UDMA_ATTR_REQMASK);

uDMAChannelAttributeEnable(UDMA_CHANNEL_ADC0, UDMA_ATTR_USEBURST);

// Only allow burst transfers

uDMAChannelControlSet(UDMA_CHANNEL_ADC0 | UDMA_PRI_SELECT, UDMA_SIZE_16 | UDMA_SRC_INC_NONE | UDMA_DST_INC_16 | UDMA_ARB_1);

uDMAChannelControlSet(UDMA_CHANNEL_ADC0 | UDMA_ALT_SELECT, UDMA_SIZE_16 | UDMA_SRC_INC_NONE | UDMA_DST_INC_16 | UDMA_ARB_1);

uDMAChannelTransferSet(UDMA_CHANNEL_ADC0 | UDMA_PRI_SELECT, UDMA_MODE_PINGPONG, (void *)(ADC0_BASE + ADC_O_SSFIFO0), &ADC_Out1, ADC_SAMPLE_BUF_SIZE);

uDMAChannelTransferSet(UDMA_CHANNEL_ADC0 | UDMA_ALT_SELECT, UDMA_MODE_PINGPONG, (void *)(ADC0_BASE + ADC_O_SSFIFO0), &ADC_Out2, ADC_SAMPLE_BUF_SIZE);

uDMAChannelEnable(UDMA_CHANNEL_ADC0); // Enables DMA channel so it can perform transfers

}

//*****************************************************************************

//

// Configure the UART and its pins.  This must be called before UARTprintf().

//

//*****************************************************************************

void

ConfigureUART(void)

{

//

// Enable the GPIO Peripheral used by the UART.

//

MAP_SysCtlPeripheralEnable(SYSCTL_PERIPH_GPIOA);

//

// Enable UART0

//

MAP_SysCtlPeripheralEnable(SYSCTL_PERIPH_UART0);

//

// Configure GPIO Pins for UART mode.

//

MAP_GPIOPinConfigure(GPIO_PA0_U0RX);

MAP_GPIOPinConfigure(GPIO_PA1_U0TX);

MAP_GPIOPinTypeUART(GPIO_PORTA_BASE, GPIO_PIN_0 | GPIO_PIN_1);

//

// Use the internal 16MHz oscillator as the UART clock source.

//

UARTClockSourceSet(UART0_BASE, UART_CLOCK_PIOSC);

//

// Initialize the UART for console I/O.

//

UARTStdioConfig(0, 115200, 16000000);

}

- Block diagram and/or Schematics showing the components, pins used, and interface.

Pin used are highlighted.

- Screenshots of the IDE, physical setup, debugging process - Provide screenshot of successful compilation, screenshots of registers, variables, graphs, etc.

- Declaration

I understand the Student Academic Misconduct Policy - http://studentconduct.unlv.edu/misconduct/policy.html

“This assignment submission is my own, original work”.

Xianjie Cao