# CPE403 – Advanced Embedded Systems

# Design Assignment #2

DO NOT REMOVE THIS PAGE DURING SUBMISSION:

Name: Xianjie Cao

Email: caox2@unlv.nevada.edu

Github Repository link (root): https://github.com/c1029324620/Flat-White

Youtube Playlist link (root): https://www.youtube.com/playlist?list=PLY90fbcjLcrnosJGw9U__oC1jxRqwp8w8

Follow the submission guideline to be awarded points for this Assignment.

Submit the following for all Assignments:

- In the document, for each task submit the modified or included code (from the base code) with highlights and justifications of the modifications. Also include the comments. If no base code is provided, submit the base code for the first task only.

- Create a private Github repository with a random name (no CPE/403, Lastname, Firstname). Place all labs under the root folder TIVAC, sub-folder named Assignment1, with one document and one video link file for each lab, place modified c files named as asng_taskxx.c.

- If multiple c files or other libraries are used, create a folder asng1_t01 and place these files inside the folder.

- The folder should have a) Word document (see template), b) source code file(s) with startup_ccs.c and other include files, c) text file with youtube video links (see template).

- Submit the doc file in canvas before the due date. The root folder of the github assignment directory should have the documentation and the text file with youtube video links.

- Organize your youtube videos as playlist under the name “cpe403”. The playlist should have the video sequence arranged as submission or due dates.

- Only submit pdf documents. Do not forget to upload this document in the github repository and in the canvas submission portal.

- Code for Tasks. for each task submit the modified or included code (from the base code) with highlights and justifications of the modifications. Also include the comments. If no base code is provided, submit the base code for the first task only. Use separate page for each task.

Task 1:

#include <stdbool.h>

#include <stdint.h>

#include <stdio.h>

#include <stdarg.h>

#include "inc/hw_i2c.h"

#include "inc/hw_memmap.h"

#include "inc/hw_types.h"

#include "inc/hw_gpio.h"

#include "driverlib/i2c.h"

#include "driverlib/sysctl.h"

#include "driverlib/gpio.h"

#include "driverlib/pin_map.h"

#include "utils/uartstdio.h"

#include "driverlib/uart.h"

#include "math.h"

#include "IQmath/IQmathlib.h"

#define TMP006_ADDR (0x40)

#define TMP006_ID   (0xFF)

#define M_ID    (0xFE)

#define TMP006_COMMAND_BIT (0x01)

#define V_OBJ   (0x00)

#define T_DIE   (0x01)

#define TMP006_CONFIG   (0x02)

#define ONE 1.0

#define LSB 1.5625e-7

#define S0 6e-5

#define A1 1.75e-3

#define A2 -1.678e-5

#define B0 -2.94e-5

#define B1 -5.7e-7

#define B2 4.63e-9

#define C2 13.4

#define T_REF 25.13

uint16_t XOR = 0xFFFF;

uint16_t TMP006_Voltage;

uint16_t TMP006_Temp;

double temp;

double TDIE;

double F;

double S;

void ConfigureUART0(void)

//Configures the UART to run at 115200 baud rate

{

// ENABLE PERIPHERAL UART 0

SysCtlPeripheralEnable(SYSCTL_PERIPH_UART0);

SysCtlPeripheralEnable(SYSCTL_PERIPH_GPIOA);

// ENABLE GPIO PORT A,FOR UART

GPIOPinConfigure(GPIO_PA0_U0RX); // PA0 IS CONFIGURED TO UART RX

GPIOPinConfigure(GPIO_PA1_U0TX); // PA1 IS CONFIGURED TO UART TX

GPIOPinTypeUART(GPIO_PORTA_BASE, GPIO_PIN_0 | GPIO_PIN_1);

UARTClockSourceSet(UART0_BASE, UART_CLOCK_PIOSC);

UARTStdioConfig(0, 115200, 16000000);

}

void I2C_Init()

{

// Enable I2C0 peripheral

SysCtlPeripheralEnable(SYSCTL_PERIPH_I2C0);

SysCtlPeripheralReset(SYSCTL_PERIPH_I2C0);

// Enable GPIO Port B to be used for I2C0

SysCtlPeripheralEnable(SYSCTL_PERIPH_GPIOB);

GPIOPinConfigure(GPIO_PB2_I2C0SCL);

GPIOPinConfigure(GPIO_PB3_I2C0SDA);

// Configure the pin muxing for I2C0 functions on Port B2 and B3

GPIOPinTypeI2CSCL(GPIO_PORTB_BASE, GPIO_PIN_2);

GPIOPinTypeI2C(GPIO_PORTB_BASE, GPIO_PIN_3);

I2CMasterInitExpClk(I2C0_BASE, SysCtlClockGet(), false);

// Set write mode

UARTprintf("I2C Master communication with Slave Address\n");

SysCtlDelay(10000);

UARTprintf("I2C Init complete!\n");

SysCtlDelay(5000000);

}

void I2C0_Send16(uint8_t slave_addr, uint8_t pointer_reg, uint16_t TxData)

{

uint8_t data;

I2CMasterSlaveAddrSet(I2C0_BASE, slave_addr, false);

I2CMasterDataPut(I2C0_BASE, pointer_reg);

I2CMasterControl(I2C0_BASE, I2C_MASTER_CMD_BURST_SEND_START);

while(I2CMasterBusy(I2C0_BASE));

// MSB First

data = (uint8_t)((TxData >> 8) & 0x00FF);

I2CMasterDataPut(I2C0_BASE, data);

while(I2CMasterBusy(I2C0_BASE));

//LSB Later

data = (uint8_t)(TxData  & 0x00FF);

I2CMasterDataPut(I2C0_BASE, data);

I2CMasterControl(I2C0_BASE, I2C_MASTER_CMD_BURST_SEND_FINISH);

while(I2CMasterBusy(I2C0_BASE));

}

uint16_t I2C_Read(uint8_t slave_addr, uint8_t pointer_reg)

{

uint8_t UpperByte = 0;

uint8_t LowerByte = 0;

uint16_t Data = 0;

// Specify Slave device address to write to

I2CMasterSlaveAddrSet(I2C0_BASE, slave_addr, false);

// Send Register address on Slave device

I2CMasterDataPut(I2C0_BASE, pointer_reg);

// Initiate send of register address from Master to Slave

I2CMasterControl(I2C0_BASE, I2C_MASTER_CMD_SINGLE_SEND);

while(I2CMasterBusy(I2C0_BASE));

// Set read mode

I2CMasterSlaveAddrSet(I2C0_BASE, slave_addr, true);

// Get first byte from slave and ack for more

I2CMasterControl(I2C0_BASE, I2C_MASTER_CMD_BURST_RECEIVE_START);

while(I2CMasterBusy(I2C0_BASE));

UpperByte = I2CMasterDataGet(I2C0_BASE);

// Get second byte from slave and nack for complete

I2CMasterControl(I2C0_BASE,  I2C_MASTER_CMD_BURST_RECEIVE_CONT);

while(I2CMasterBusy(I2C0_BASE));

LowerByte = I2CMasterDataGet(I2C0_BASE);

Data = (UpperByte<<8) | LowerByte;

return Data;

}

int main(void)

{

SysCtlClockSet(SYSCTL_SYSDIV_4 | SYSCTL_USE_PLL | SYSCTL_XTAL_16MHZ | SYSCTL_OSC_MAIN);

ConfigureUART0();

I2C_Init();

double IQ_NUM_Mul_Const = pow(2, -24);

_iq24 v_obj, t_die, s0, a1, a2, t_ref, b0, b1, b2, c2, s, t_die_sub_t_ref, v_os, f_v_obj, v_obj_sub_v_os, one;

_iq24 lsb;

s0 = _IQ24(S0);

a1 = _IQ24(A1);

a2 = _IQ24(A2);

t_ref = _IQ24(T_REF);

b0 = _IQ24(B0);

b1 = _IQ24(B1);

b2 = _IQ24(B2);

c2 = _IQ24(C2);

lsb = _IQ24(LSB);

one = _IQ24(ONE);

while(1)

{

TMP006_Voltage = I2C_Read(TMP006_ADDR, V_OBJ);

if(TMP006_Voltage > 0x7FFF)     //full scale from 0 to 7FFFh, if the reading is negative, get its absolute value.

{

//UARTprintf("negative\n");

TMP006_Voltage = (TMP006_Voltage ^ XOR) + 1;

}

//TMP006_Voltage = TMP006_Voltage;

UARTprintf("I2C V_OBJ: %d\n", TMP006_Voltage);

TMP006_Temp = (I2C_Read(TMP006_ADDR, T_DIE)>>2)/32 ; //Converting the integer temperature result of the TMP006

//to physical temperature is done by rightshifting the last two LSBs

t_die = _IQ24(TMP006_Temp);                             //followed by a divide-by-32 of TREG to obtain the physical temperature result in degreesCelsius. T

v_obj = _IQ24(TMP006_Voltage);

v_obj = _IQ24mpy(v_obj, lsb);

UARTprintf("I2C T_DIE: %d C\n", TMP006_Temp);

t_die_sub_t_ref = t_die - t_ref;

s = _IQ24mpy(s0,(_IQ24mpy(a1, t_die_sub_t_ref) + _IQ24mpy(a2, (_IQ24mpy(t_die_sub_t_ref, t_die_sub_t_ref))) + one));

v_os = _IQ24mpy(t_die_sub_t_ref, b1) + _IQ24mpy(b2,(_IQ24mpy(t_die_sub_t_ref, t_die_sub_t_ref))) + b0;

v_obj_sub_v_os = v_obj - v_os;

f_v_obj = v_obj_sub_v_os + _IQ24mpy(c2,_IQ24mpy(v_obj_sub_v_os, v_obj_sub_v_os));

//Convert to double.

F = f_v_obj * IQ_NUM_Mul_Const;//convert to double

S = s * IQ_NUM_Mul_Const;

TDIE = t_die * IQ_NUM_Mul_Const;

temp = pow((pow(TDIE, 4.0) + F/S), 0.25);

SysCtlDelay(5000000);

}

}

- Block diagram and/or Schematics showing the components, pins used, and interface.

Components used: TM4C123GXl, BoosterPack MKII, TMP006 sensor, Jump wires, code composer studio.

Pins used: 3.3V, 5V, GND, PB2, PB3.

Interface: I2C

- Screenshots of the IDE, physical setup, debugging process - Provide screenshot of successful compilation, screenshots of registers, variables, graphs, etc.

- Declaration

I understand the Student Academic Misconduct Policy - http://studentconduct.unlv.edu/misconduct/policy.html

“This assignment submission is my own, original work”.

Xianjie Cao