# CPE403 – Advanced Embedded Systems

# Design Assignment #3

DO NOT REMOVE THIS PAGE DURING SUBMISSION:

Name: Xianjie Cao

Email: Caox2@unlv.nevada.edu

Github Repository link (root): https://github.com/c1029324620/Flat-White.git

Youtube Playlist link (root): https://www.youtube.com/playlist?list=PLY90fbcjLcrnosJGw9U__oC1jxRqwp8w8

Follow the submission guideline to be awarded points for this Assignment.

Submit the following for all Assignments:

- In the document, for each task submit the modified or included code (from the base code) with highlights and justifications of the modifications. Also include the comments. If no base code is provided, submit the base code for the first task only.

- Create a private Github repository with a random name (no CPE/403, Lastname, Firstname). Place all labs under the root folder TIVAC, sub-folder named Assignment1, with one document and one video link file for each lab, place modified c files named as asng_taskxx.c.

- If multiple c files or other libraries are used, create a folder asng1_t01 and place these files inside the folder.

- The folder should have a) Word document (see template), b) source code file(s) with startup_ccs.c and other include files, c) text file with youtube video links (see template).

- Submit the doc file in canvas before the due date. The root folder of the github assignment directory should have the documentation and the text file with youtube video links.

- Organize your youtube videos as playlist under the name “cpe403”. The playlist should have the video sequence arranged as submission or due dates.

- Only submit pdf documents. Do not forget to upload this document in the github repository and in the canvas submission portal.

- Code for Tasks. for each task submit the modified or included code (from the base code) with highlights and justifications of the modifications. Also include the comments. If no base code is provided, submit the base code for the first task only. Use separate page for each task.

*  ======== empty_min.c ========

*/

/* XDCtools Header files */

#include <xdc/std.h>

#include <xdc/runtime/System.h>

#include <xdc/runtime/Log.h>                //needed for any Log_info() call

/* BIOS Header files */

#include <ti/sysbios/BIOS.h>

#include <ti/sysbios/knl/Task.h>

/* TI-RTOS Header files */

#include <ti/drivers/GPIO.h>

#include <ti/drivers/PWM.h>

// #include <ti/drivers/I2C.h>

// #include <ti/drivers/SDSPI.h>

// #include <ti/drivers/SPI.h>

#include <ti/drivers/UART.h>

// #include <ti/drivers/Watchdog.h>

// #include <ti/drivers/WiFi.h>

/* Board Header file */

#include "Board.h"

//------------------------------------------

// TivaWare Header Files

//------------------------------------------

#include <stdint.h>

#include <stdbool.h>

#include <stdlib.h>

#include "inc/hw_types.h"

#include "inc/hw_memmap.h"

#include "driverlib/sysctl.h"

#include "driverlib/gpio.h"

#include "inc/hw_ints.h"

#include "driverlib/interrupt.h"

#include "driverlib/timer.h"

#include "driverlib/adc.h"

#define TASKSTACKSIZE   512

#define TASKSTACKSIZE2  768

Task_Struct task0Struct, task1Struct, task2Struct;

Char task0Stack[TASKSTACKSIZE], task1Stack[TASKSTACKSIZE], task2Stack[TASKSTACKSIZE2];

Task_Handle task;

uint16_t adcValue;

uint16_t duty = 0;

uint8_t counter = 0;

uint32_t ui32ADCValue[1];

UART_Handle uart;

UART_Params uartParams;

char buffer[20];

UART_Handle uart;

UART_Params uartParams;

const char echoPrompt[] = "\fUART Initialized!!\r\n";

const char writeDone[] = "ADC Value: ";

/*

*  ======== heartBeatFxn ========

*  Toggle the Board_LED0. The Task_sleep is determined by arg0 which

*  is configured for the heartBeat Task instance.

*/

Void heartBeatFxn(UArg arg0, UArg arg1)

{

while (1) {

Task_sleep((UInt)arg0);

GPIO_toggle(Board_LED2);    //PF1, RED LED

}

}

void pwmFxn(UArg arg0, UArg arg1)

{

PWM_Handle pwm1;

PWM_Params pwmParams;

uint16_t pwmPeriod = 4096;      //period and duty in ms

PWM_Params_init(&pwmParams);

pwmParams.period = pwmPeriod;

pwm1 = PWM_open(Board_PWM0, &pwmParams);

if(pwm1 == NULL)

{

System_abort("Board_PWM0 did not open");

}

//updating the duty cycle.

while(1)

{

PWM_setDuty(pwm1, duty);

Task_sleep((UInt) arg0);

}

}

void buttonFxn(unsigned int index)

{

//callback function to the button 0 interrupt

duty = adcValue;

}

Void echoFxn(UArg arg0, UArg arg1)

{

/* Create a UART with data processing off. */

UART_Params_init(&uartParams);

uartParams.writeDataMode = UART_DATA_BINARY;

uartParams.readDataMode = UART_DATA_BINARY;

uartParams.readReturnMode = UART_RETURN_FULL;

uartParams.readEcho = UART_ECHO_OFF;

uartParams.baudRate = 9600;

uart = UART_open(Board_UART0, &uartParams);

if (uart == NULL) {

System_abort("Error opening the UART");

}

UART_write(uart, echoPrompt, sizeof(echoPrompt));

/* Loop forever echoing */

while(1){

sprintf(buffer, "%i\r\n", adcValue);

UART_write(uart, writeDone, sizeof(writeDone));

UART_write(uart, buffer, sizeof(buffer));

}

}

void hardware_init(void)

{

uint32_t ui32Period;

//Set CPU Clock to 40MHz. 400MHz PLL/2 = 200 DIV 5 = 40MHz

//SysCtlClockSet(SYSCTL_SYSDIV_5|SYSCTL_USE_PLL|SYSCTL_XTAL_16MHZ|SYSCTL_OSC_MAIN);

// Timer 2 setup code

SysCtlPeripheralEnable(SYSCTL_PERIPH_TIMER2);           // enable Timer 2 periph clks

TimerConfigure(TIMER2_BASE, TIMER_CFG_PERIODIC);        // cfg Timer 2 mode - periodic

ui32Period = (SysCtlClockGet() /1000);                     // period = CPU clk div 1000 (1ms)

TimerLoadSet(TIMER2_BASE, TIMER_A, ui32Period);         // set Timer 2 period

TimerIntEnable(TIMER2_BASE, TIMER_TIMA_TIMEOUT);        // enables Timer 2 to interrupt CPU

TimerEnable(TIMER2_BASE, TIMER_A);                      // enable Timer 2

//set up ADC

//using the A11.

SysCtlPeripheralEnable(SYSCTL_PERIPH_GPIOB);

GPIOPinTypeADC(GPIO_PORTE_BASE, GPIO_PIN_5);

SysCtlPeripheralEnable(SYSCTL_PERIPH_ADC0);

ADCHardwareOversampleConfigure(ADC0_BASE, 32);

ADCSequenceConfigure(ADC0_BASE, 0, ADC_TRIGGER_PROCESSOR, 0); // Changed to sequencer #2

ADCSequenceStepConfigure(ADC0_BASE, 0, 0, ADC_CTL_CH11|ADC_CTL_IE|ADC_CTL_END);

ADCIntEnable(ADC0_BASE, 0);

ADCSequenceEnable(ADC0_BASE, 0);

}

/*

*  ======== main ========

*/

int main(void)

{

Task_Params taskParams;

Task_Params pwmTask;

Task_Params uartTask;

/* Call board init functions */

Board_initGeneral();

Board_initGPIO();

Board_initPWM();

hardware_init();

// Board_initI2C();

// Board_initSDSPI();

// Board_initSPI();

Board_initUART();

// Board_initUSB(Board_USBDEVICE);

// Board_initWatchdog();

// Board_initWiFi();

Task_Params_init(&taskParams);

taskParams.arg0 = 500;

taskParams.stackSize = TASKSTACKSIZE;

taskParams.stack = &task0Stack;

Task_construct(&task0Struct, (Task_FuncPtr)heartBeatFxn, &taskParams, NULL);

Task_Params_init(&pwmTask);

pwmTask.arg0 = 50;

pwmTask.stackSize = TASKSTACKSIZE;

pwmTask.stack = &task1Stack;

Task_construct(&task1Struct, (Task_FuncPtr)pwmFxn, &pwmTask, NULL);

task = Task_handle(&task1Struct);

Task_Params_init(&uartTask);

uartTask.stackSize = TASKSTACKSIZE2;

uartTask.stack = &task2Stack;

uartTask.instance->name = "echo";

Task_construct(&task2Struct, (Task_FuncPtr)echoFxn, &uartTask, NULL);

/* Turn on user LED  */

// GPIO_write(Board_LED2, Board_LED_ON);

/* Start BIOS */

BIOS_start();

return (0);

}

void timer2_ISR(voide)

{

TimerIntClear(TIMER2_BASE, TIMER_TIMA_TIMEOUT);         // must clear timer flag FROM timer

ADCIntClear(ADC0_BASE, 0);

ADCProcessorTrigger(ADC0_BASE, 0);

counter++;

if(counter == 5)

{

//perform ADC read

ADCSequenceDataGet(ADC0_BASE, 0, ui32ADCValue);

adcValue = ui32ADCValue[0];

}

if(counter == 10)

{

//print ADC value to the UART

//

}

if(counter ==15)

{

//read the value of the button 0. and print the status to the console.

if(GPIO_read(Board_BUTTON0) == 1)

{

System_printf("Button 0 not pressed\n");

System_flush();

}

else

{

System_printf("Button 0 is pressed\n");

System_flush();

}

GPIO_setCallback(Board_BUTTON0, buttonFxn);

GPIO_enableInt(Board_BUTTON0);

//reset the counter.

counter = 0;

}

}

- Block diagram and/or Schematics showing the components, pins used, and interface.

Components used: TM4C123GXL, BoosterPack MKII, jump wires, code composer studio.

Pins used: 3.3V, 5V, GND, PF1, PF2, PA0, PA1, PB5.

Interface: UART, ADC.

- Screenshots of the IDE, physical setup, debugging process - Provide screenshot of successful compilation, screenshots of registers, variables, graphs, etc.

- Declaration

I understand the Student Academic Misconduct Policy - http://studentconduct.unlv.edu/misconduct/policy.html

“This assignment submission is my own, original work”.

Xianjie Cao